This routine is to be used only for actual data. It is senseless in the Monte Carlo framework.

accepted {\it Detector\_name's} with corresponding data types (in the envelopping detector blocklet for

module blocklets) are listed

here ({\bf numbers are in hex}):

\par

\begin{tabular}{|cc|cc|cc|cc|cc|}

\hline

"DCH"   & 1 & "TRD"  & 2 & "PRS"  & 3 & "CAL"  & 4  &  "MUO" & 5  \\

\hline

"SCI"  & 6  & "VET"   & 7 & "FCA"  & 8 & "HCA"  & 9 &        &  \\

\hline

"DCHC"  & 11 & "TRDC"  & 12 & "PRSC"  & 13 & "CALC"  & 14  &  "MUOC" & 15  \\

\hline

"SCIC"  & 16  & "VETC"   & 17 & "FCAC"  & 18 & "HCAC"  & 19 &   &    \\

\hline

"DCHM"   & 21 & "TRDM"  & 22 & "PRSM"  & 23 & "CALM"  & 24  &  "MUOM" & 25  \\

\hline

"SCIM"  & 26  & "VETM"   & 27 & "FCAM"  & 28 & "HCAM"  & 29 &        &  \\

\hline

"DCHS"   & 31 & "TRDS"  & 32 & "PRSS"  & 33 & "CALS"  & 34  &  "MUOS" & 35  \\

\hline

"SCIS"  & 36  & "VETS"   & 37 & "FCAS"  & 38 & "HCAS"  & 39 &  "BASS"   & 3a  \\

\hline

"BMON" & 40  & "BMOF"     & 41 & "SCAL"  & 5f & "SLWC"  & 30   &           &  \\

\hline

\end{tabular}

\par

{\it Module\_name} specifies the FB id of the data blocklet requested. It

can be: "CADC" (charge ADC), "PADC" (Peak ADC), "TDC" (LeCroy TDC's)

corresponding repectively to fastbus id's 0x6904, 0x6905 and 0x103E. Also

allowed values are "NBC" (fb=1), "NFMI" (fb=2), "NFMO" (fb=1) for

beam information in and out of spill, "MSCN" (fb =2), "SCAL" (fb=1),

"ERLG" (fb =2), "STAT" (fb=3). A string containing a decimal value such as "12"

will also be properly decoded.

{\it Where} is an array reserved with dimension Length to be provided by user.

On input,

*{\it Length} may either be {\it Where} length or 0. If *{\it Length} and contains the

data length on output it will be set to the data length and data will contain

the data. The

user misses part of the data if {\it Length} is equal on input and output (the get\_data\_array

function provides a way to avoid that). If *{\it Length} is equal to 0 on input then on output

it contains the data length and *{\it Where} contains the offset with respect to the

Rawd bank. From fortran the data can thus be copied to a private array and from C the

function get\_data\_array may be used to retrieve the data in {\it Where}.

The routine returns -1 if the data is not there or the format is incorrect, with

printed error messages. A positive result is the number of words that cannot

be interpreted as Fastbus words comming from ADC's or TDC's. Returns 0 if  everything OK. \\

Here is an example of how to use this routine both from fortran and from C programs. In this example the data for the calorimeter blocklet, module `PADC' are retrieved~:

\begin{verbatim}

C>

#define SIZEMAX 1000

int length, data[SIZEMAX];

length = SIZEMAX;

ierr = get_data("CAL","PADC", data, &length);

...

/* the data words are accessed as data[i] */

...

C>

int length, where, ierr,i;

int *data;

length = 0; /* mandatory to tell get_data not to copy data in where */

ierr = get_data( "CAL", "PADC", &where, &length)

if (ierr == 0) data = get_data_array(where, length);

....

/* the data words are accessed as data[i] */

....

/* mandatory since get_data_array performs a dynamic allocation */

free(data);

Fortran>

#include nzbank.inc

INTEGER*4 FGET_DATA

EXTERNAL FGET_DATA

INTEGER *4 IERR, LEN, OFFSET

...

LEN = 0

IERR = FGET_DATA('CAL','PADC',OFFSET,LEN)

CALL MZWORK(INOMSTO,NOMSTO,NOMSTO(LEN),0)

CALL UCOPY(IQ(LRAWD+OFFSET+1),NOMSTO,LEN)

...

\end{verbatim}